WWW.VUSR.NET

FINALTERM  EXAMINATION

Spring 2010

MGT613- Production / Operations Management

Question No: 1    ( Marks: 1 )    - Please choose one

Which one of the following is a measure of productivity that involves measuring inputs and outputs using a common unit of measurement?

► Multifactor

► Partial

► Single

► Total

Question No: 2    ( Marks: 1 )    - Please choose one

Which of the following is NOT usually considered a general characteristic of a service?

► Production and sales cannot easily be separated functionally

► Many services involve both tangible and intangible outputs

► Production and consumption can always be separated

► Degree of customer contact is high

Question No: 3    ( Marks: 1 )    - Please choose one

Which of the following statements corresponds to an order-winning characteristic?

► A factor which may be significant in other parts of the organization

► A factor which gives an organization a competitive edge

► A factor which serves as a minimum standard for purchase

► A factor which increases the profitability of the organization

Question No: 4    ( Marks: 1 )    - Please choose one

Which one of the following correctly explains the elements of a good forecast?

► Timely, having a purpose, accurate, written, reliable, meaningful

► Timely, accurate, judgmental, correctness, verbal, simple to use

► Timely, accurate, reliable, meaningful, written, simple to use

► Timely, accurate, reliable, consistent, meaningful, written, simple to use

Question No: 5    ( Marks: 1 )    - Please choose one

Which one of the following factors provides a basis for comparing alternative forecasting techniques?

► Time dimension

► Reliability factor

► Degree of accuracy

► Measuring units

Question No: 6    ( Marks: 1 )    - Please choose one

Which one of the following is an outcome of MINIMAX regret?

► The best of the worst possible payoff

► The best possible payoff

► The best average payoff

► The least of the worst regret

Question No: 7    ( Marks: 1 )    - Please choose one

Which one of the following is NOT a step of decision making process?

► Develop alternatives

► Monitor results

► Consider risk averseness strategies

► Specify criteria for decision

Question No: 8    ( Marks: 1 )    - Please choose one

Effective capacity is influenced by all of the following factors EXCEPT:

► Facilities

► Product mix

► Processes

► Operations

Question No: 9    ( Marks: 1 )    - Please choose one

Which one of the following defines the range of the smoothing constant (Alpha)?

► 2 and -2

► 1 and -1

► 0 and -1

► 0 and 1

Question No: 10    ( Marks: 1 )    - Please choose one

In exponential smoothing model, α denotes:

► Smoothing constant

► Actual forecast

► Forecast error

► Previous forecast

Question No: 11    ( Marks: 1 )    - Please choose one

Which one of the following refers to using some of the components of old products in the manufacturing of new products?

► Manufacturability

► Remanufacturing

► Robust design

► Automation

Question No: 12    ( Marks: 1 )    - Please choose one

Which one of the following refers to a team having representatives from different functional areas of the organization?

► Traditional work group

► Self-directed team

► Cross-functional team

► Self motivated team

Question No: 13    ( Marks: 1 )    - Please choose one

Who introduced the use of statistical control charts to Japanese manufacturers?

► Edwards Deming

► Joseph Juran

► Kaoru Ishikawa

► Philip Crosby

Question No: 14    ( Marks: 1 )    - Please choose one

Mr. Ali purchased a TV set. After a period of a year, the picture quality started deteriorating. He went to the company and complained and the company responded subsequently. Which of the following dimensions of quality would come into play?

► Reliability

► Conformance

► Serviceability

► Aesthetics

Question No: 15    ( Marks: 1 )    - Please choose one

Who proposed the concept of “total quality control” in the mid 1950s?

► Armand Feigenbaum

► G.S Radford

► W. Shewhart

► David Gravin

Question No: 16    ( Marks: 1 )    - Please choose one

Which of the following is a pair of service quality dimensions?

► Reliability and Responsiveness

► Uniqueness and Universality

► Specification and Security

► Reliability and Reputation

Question No: 17    ( Marks: 1 )    - Please choose one

Which of the following best describes the ISO?

► International Organization for Statements

► International Operations for Statements

► International Organization for Standardization

► International Operations for Statements

Question No: 18    ( Marks: 1 )    - Please choose one

Which of the following stage of Deming's wheel involves evaluating the improvement plan?

► Plan

► Do

► Check

► Act

Question No: 19    ( Marks: 1 )    - Please choose one

What would be the total cost of inventory, if a firm holds 1000 units of a product ‘A’, where the carrying cost is Rs. 5 per unit?

► Rs. 995

► Rs. 5000

► Rs. 1050

► Rs. 200

Question No: 20    ( Marks: 1 )    - Please choose one

The optimal order quantity shows a trade-off between which one of the following sets of costs?

► Carrying costs and shortage costs

► Carrying costs and ordering costs

► Carrying costs and stockout costs

► Carrying costs and setup costs

Question No: 21    ( Marks: 1 )    - Please choose one

Which one of the following mathematical expressions can be employed to compute annual carrying cost?

► (Q-2)H

► (Q+2)H

► (Q÷2)H

► (Q×2)/H

Question No: 22    ( Marks: 1 )    - Please choose one

Which of the following is used to store information about the status of each item with respect to time period?

► Master schedule

► Bill of materials file

► Inventory records file

► Rough – cut capacity plan

Question No: 23    ( Marks: 1 )    - Please choose one

Which of the following compares the known and estimated future capacity requirements?

► Capacity reports

► Operations reports

► Load reports

► Inventory records

Question No: 24    ( Marks: 1 )    - Please choose one

Which of the following is the first step of Capacity planning?

► Initial feasibility report is prepared

► Master schedule is tested for feasibility

► Adjustments are made in master schedule before it finalizes

► Material requirements are ascertained

Question No: 25    ( Marks: 1 )    - Please choose one

Which of the following are the priorities emphasized most often in JIT system?

► High quality and minimal wastage

► Low cost and consistent quality

► Low inventory and consistent material supply

► Low cost and low inventory

Question No: 26    ( Marks: 1 )    - Please choose one

Just in Time system maintains inventory with which of the following lot size/sizes?

► Small

► Medium

► Large

► Can be small and medium both

Question No: 27    ( Marks: 1 )    - Please choose one

Which of the following statements is TRUE about a Gantt chart?

► It represents an important event in the completion of a project

► It relates interdependent activities to their completion time

► It arranges events in order of importance

► It is used to schedule independent activities

Question No: 28    ( Marks: 1 )    - Please choose one

Network activities are used to determine all of the following EXCEPT:

► Critical path

► Budgeted cost

► Slack time

► Expected project duration

Question No: 29    ( Marks: 1 )    - Please choose one

With reference to Network activities, what does the term ‘EF’ stand for?

► Equal Finish

► Economic Finish

► Early Finish

► Easy Finish

Question No: 30    ( Marks: 1 )    - Please choose one

Which one of the following is NOT an element of a queuing system?

► Population source

► Waiting line

► Order processing

► Logistics

Question No: 31    ( Marks: 1 )    - Please choose one

Which one of the following refers to translating the voice of the customer into technical design requirements?

► Concurrent design

► Design for manufacture

► Robust design

► Quality function deployment

Question No: 32    ( Marks: 1 )    - Please choose one

Which one of the following is the goal of work sampling?

► Studying Individual human motions that are used in a job task

► Determining the length of time it will take to undertake a particular task

► Determining the amount of time a worker spends on various activities

► Specifying the content and methods to perform a job

Question No: 33    ( Marks: 1 )    - Please choose one

Training program and quality audits come under which type of costs that an organization has to bear as cost of service quality?

► Internal failure cost

► Recovery cost

► Prevention cost

► Detection cost

Question No: 34    ( Marks: 1 )    - Please choose one

Which one of the following is the most important criteria in selecting winners for a quality award?

► Strong motivation

► Quality management practices

► Employee empowerment

► Business performance results

Question No: 35    ( Marks: 1 )    - Please choose one

Which of the following is NOT a disadvantage of excessive overtime?

► Decline quality of work

► Work schedules become well-defined

► 150 percent expensive of the regular time-pay rate

► Low Productivity

Question No: 36    ( Marks: 1 )    - Please choose one

A company mostly involved in transportation and shipping costs than construction and land costs is probably part of which of the following industries?

► Heavy manufacturing

► Retail and service

► Warehouse and distribution

► Light industry

Question No: 37    ( Marks: 1 )    - Please choose one

A company is operating under a continuous review system. It has an average demand of 50 units per week for the item it produces. The lead time for the item is 6 weeks, and it costs the company $30 to process each order. The holding cost for each unit is $10 per year. The company operates 52 weeks per year. What is the economic order quantity (EOQ) for this item?

► Greater than 175 units but less than or equal to 200 units

► Less than 175 units

► Greater than 200 units but less than or equal to 230 units

► Greater than 230 units

Question No: 38    ( Marks: 1 )    - Please choose one

Perpetual system of inventory management is also known as which of the following?

► First-in-first-out

► Probability model

► Fixed-order quantity

► Periodic review

Question No: 39    ( Marks: 1 )    - Please choose one

If a manager wants to see the effect of change in Master Production Schedule (MPS) on purchasing requirements for certain suppliers, which of the following system would help in this regard?

► Material Resource Planning system (MRP)

► Enterprise Resource Planning system (ERP)

► Capacity Requirement Planning system (CRP)

► Manufacturing Resource Planning system (MRPII)

Question No: 40    ( Marks: 1 )    - Please choose one

If alpha = 0, how many containers are needed with the given data?

K = 1500 (0.8 + 0.2) (1 + 0)

270

► 6.5 Containers

► 7 Containers

► 6 Containers

► 9 Containers

Question No: 41    ( Marks: 1 )    - Please choose one

Eliminating disruptions and excess inventory are _____________ in Just-In-Time production systems.

► Tactical goals

► Strategic goals

► Secondary goals

► Primary goals

Question No: 42    ( Marks: 1 )    - Please choose one

In supply chain management Electronic Data Interchange (EDI) and Bar coding is a characteristic of which of the following?

► Logistics

► Technology

► Autonomation

► Just-In-Time System

Question No: 43    ( Marks: 1 )    - Please choose one

In labor-limited environment, in which resource constraint is the amount of worker available, which of the following is the best option for effective scheduling in a manufacturing firm?

► Hire more skilled labor for the job to be completed

► Workers should be trained to work on a variety of machines

► Assign labor to the next workstation irrespective of their skills

► More machines should be installed for flexibility of operations

Question No: 44    ( Marks: 1 )    - Please choose one

Tasks are easily scheduled in which of the following production environments?

► Intermediate-volume-systems

► Low-to-medium volume production (Job shop)

► Medium-to-high volume production (Flow shop)

► Optimal production mixes

Question No: 45    ( Marks: 1 )    - Please choose one

Which of the following is the necessary qualification for a project manager?

► PMP certification

► PMD certification

► FMA certification

► CMP certification

Question No: 46    ( Marks: 1 )    - Please choose one

Queuing analysis aims to minimize which of the given costs?

► Both customer waiting cost and service capacity cost

► None of the given options

► Customer waiting cost

► Service capacity cost

Question No: 47    ( Marks: 1 )    - Please choose one

Which of the following is a cause of development of waiting lines?

► Customer arrival rate varies through out the day

► Time required to process a customer may vary

► Some transactions are complicated and require above average process time

► All of the given options

Question No: 48    ( Marks: 1 )    - Please choose one

During the period of high unemployment or when low-skilled labor is acceptable, which of the following strategy would be cost effective?

► Chase demand strategy

► Level capacity strategy

► Proactive strategy

► Mixed or hybrid strategies

Question No: 49    ( Marks: 3 )

As an operations manager of a firm what significant issues you have to consider while implementing supply chain management?

Question No: 50    ( Marks: 3 )

How would you reveal the importance of maintaining good relationship with suppliers in a JIT system?

.

Question No: 51    ( Marks: 5 )

As operations manager of a manufacturing firm, how can you judge the effectiveness of inventory management?

Question No: 52    ( Marks: 5 )

What is bullwhip effect? What are its consequences?

Question No: 53    ( Marks: 5 )

In the above bill of materials, if existing inventory =0, then how many units of G, E, F, H and B must be purchased to produce 4 units of A?

WWW.VUSR.NET