www.VUSR.net

FINALTERM  EXAMINATION

Spring 2010

CS601- Data Communication

Ref No:

Time: 90 min

Marks: 60

| Student Info |  |  |  |
| --- | --- | --- | --- |
| Student ID: |  |  |  |
| Center: |  |  |  |
| Exam Date: |  |  |  |

| For Teacher's Use Only |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q No. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | Total |
| Marks |  |  |  |  |  |  |  |  |  |
| Q No. | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |  |
| Marks |  |  |  |  |  |  |  |  |  |
| Q No. | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |  |
| Marks |  |  |  |  |  |  |  |  |  |
| Q No. | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 |  |
| Marks |  |  |  |  |  |  |  |  |  |
| Q No. | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 |  |
| Marks |  |  |  |  |  |  |  |  |  |

Question No: 1      ( Marks: 1 ) - Please choose one

When data are transmitted from device A to device B, the header from A’s layer 4 is read by B’s __________ layer.

► physical

► transport

► application

► none of the given

Question No: 2      ( Marks: 1 ) - Please choose one

A periodic signal completes one cycle in 0.001 s. What is the frequency?

► 1 Hz

► 100 Hz

► 1 KHz

► 1 MHz

Question No: 3      ( Marks: 1 ) - Please choose one

Zero crossing bandwidth is also called as equivalent noise bandwidth.

► True

► False

Question No: 4      ( Marks: 1 ) - Please choose one

Amplitude in ASK is more resistive to EMI and Noise.

► True

► False

Question No: 5      ( Marks: 1 ) - Please choose one

If FCC regulations are followed, the carrier frequencies of adjacent AM radio stations are ____________ apart.

► 5 KHz

► 10 KHz

► 200 KHz

► 530 KHz

Question No: 6      ( Marks: 1 ) - Please choose one

There are _________________ types of serial transmission:

► 1

► 2

► 3

► 4

Question No: 7      ( Marks: 1 ) - Please choose one

Synchronous transmission have _____________.

► a start bit

► a stop bit

► ga

ps between bytes

► none of the given

Question No: 8      ( Marks: 1 ) - Please choose one

Which of the following is an example of ITU-T modem standards:

► T-series

► X-series

► N-series

► V-series

Question No: 9      ( Marks: 1 ) - Please choose one

The maximum data rate in the uploading direction is still ____________.

► 26.6 Kbps

► 33.6 Kbps

► 36.6 Kbps

► 46.6 Kbps

Question No: 10      ( Marks: 1 ) - Please choose one

Category 5 UTP cable is used for data transmission of upto__________.

► 100 Mbps

► 200 Mbps

► 250 Mbps

► 400 Mbps

Question No: 11      ( Marks: 1 ) - Please choose one

All of popular Fiber optic connectors are ___________ shaped.

► Conical

► Barrel

► Circular

► Rectangular

Question No: 12      ( Marks: 1 ) - Please choose one

The VLF and LF bands use _________ propagation for communications.

► Ground

► Sky

► Line of sight

► Space

Question No: 13      ( Marks: 1 ) - Please choose one

Multiplexing is the set of techniques that allows simultaneous TX of multiple signals across  ___________ data link

► Single

► Multi

► Single and Multi

► none of the given

Question No: 14      ( Marks: 1 ) - Please choose one

Need for Addressing makes Asynchronous TDM inefficient for bit or byte ___________.

► Interleaving

► Addition

► Substraction

► None of the given

Question No: 15      ( Marks: 1 ) - Please choose one

_________ takes data from one high speed line and breaks it into portions.

► Multiplexing

► Inverse multiplexing

► Inverse subtraction

► Inverse addition

Question No: 16      ( Marks: 1 ) - Please choose one

A traditional telephone line has a bandwidth of __________.

► 2000Hz

► 4000 Hz

► 2000 MHz

► 4000 MHz

Question No: 17      ( Marks: 1 ) - Please choose one

If the ASCII character H is sent and the character I is received, what type of error is this?

► Single-bit

► Multiple-bit

► Burst

► Recoverable

Question No: 18      ( Marks: 1 ) - Please choose one

At the CRC generator, __________ added to the data unit after the division process

► 0s are

► 1s are

► The polynomial

► The CRC remainder is

Question No: 19      ( Marks: 1 ) - Please choose one

Error control in the data link layer is based on ________________

► automatic repeat request

► automatic repeat acknowledgment

► automatic send acknowledgment

Question No: 20      ( Marks: 1 ) - Please choose one

In line discipline the initiator first transmits a frame called an ________

► Enquiry

► Acknowledgment

► NAK

► Request

Question No: 21      ( Marks: 1 ) - Please choose one

Primary device uses ____________ to receive transmission from the secondary devices.

► ACK

► ENQ

► POLL

Question No: 22      ( Marks: 1 ) - Please choose one

Flow control is needed to prevent ___________.

► Bit errors

► Overflow of the sender buffer

► Overflow of the receiver buffer

► Collision between sender and receiver

Question No: 23      ( Marks: 1 ) - Please choose one

For stop-and-wait ARQ, for N data packets sent, ___________ acknowledgments are needed.

► N

► 2N

► N-1

► N+1

Question No: 24      ( Marks: 1 ) - Please choose one

In Y-MODEM Multiple files can be sent simultaneously

► True

► False

Question No: 25      ( Marks: 1 ) - Please choose one

HDLC is an acronym for ______________.

► High-duplex line communication

► High-level data link control

► Half-duplex digital link combination

► Host double-level circuit

Question No: 26      ( Marks: 1 ) - Please choose one

Token Bus has no commercial application in data communications

► True

► False

Question No: 27      ( Marks: 1 ) - Please choose one

The BNC-T connector is a T-shaped device with _______ ports

► Three

► Two

► Four

► Five

Question No: 28      ( Marks: 1 ) - Please choose one

Each station in the Token Ring regenerates the frame.

► True

► False

Question No: 29      ( Marks: 1 ) - Please choose one

Repeater works on __________     layer.

► Data Link

► Physical

► Network

► Application

Question No: 30      ( Marks: 1 ) - Please choose one

Trunks are transmission media such as _________ that handle the telephone to the nearest end office.

► Satellite links

► Twisted-pair & Fiber-optic

► Twisted-pair

► Fiber-optic

Question No: 31      ( Marks: 2 )

What kind of error is undetectable by the checksum? [2]

Error detection method used by the higher layers like VRC,LRC,CRC,checksum is also based on the concept of redundancy

Question No: 32      ( Marks: 2 )

What are properties of signals?

Question No: 33      ( Marks: 2 )

Whether in Asynchronous or Synchronous TDM, addressing is used?

Question No: 34      ( Marks: 2 )

What is the basic purpose of Router?

Question No: 35      ( Marks: 3 )

Why we need a Null Modem?

Question No: 36      ( Marks: 3 )

Count LRC for the following bits?

10011010  10100101   1101 0110

Question No: 37      ( Marks: 3 )

what are the categories of multiplexing?

There are three catogaries of  multiplxing

FDM

TDM

Have a two other catagories

Synchronous and asyrounce

WDM

Question No: 38      ( Marks: 3 )

What are the three purposes of control frames at data link layer?

Question No: 39      ( Marks: 5 )

Compare line decipline methods ENQ/ACK and Poll/ Select?

=>ENQ/ACK coordinates which device may start a transmission and whether or not the intended recipient is ready and enabled.

=> Using ENQ/ACK, a session can be initiated by either station on a link as long as both are of equal rank.

=> In both half-duplex and full-duplex transmission, the initiating device establishes the session.

=> In half duplex, the initiator then sends its data while the responder waits. The responder may take over the link when the initiator is finished or has requested a response.

=> In full duplex, both devices can transmit simultaneously once the session has been established. 

POLL/SELECT:

=> The poll/select method of line discipline works with topologies where one device is designated as a primary station and the other devices are secondary stations.

=> Multipoint systems must coordinate several nodes, not just two.

=> The question to be determined in these cases, therefore, is more than just, are you ready? It is also, which of the several nodes has the right to use the channel?

Question No: 40      ( Marks: 5 )

what is the difference between character oriented and bit oriented protocols?

WWW.VUSR.net